### 1. Purpose of the Test

- Evaluate Tear Production: The primary goal is to measure the rate of tear formation and evaluate the integrity of the lacrimal secretory system.

- Diagnose Dry Eye: It is the standard clinical test to assess aqueous tear deficiency. It helps differentiate between evaporative dry eye and tear-deficient dry eye (such as Sjögren’s syndrome),.

- Assess Total vs. Basic Secretion:

  - Schirmer Test I (Without Anesthetic): Measures total tear secretion, which includes both basic secretion and reflex tearing caused by the irritation of the paper strip,.

  - Basic Secretion Test (With Anesthetic): Measures only basic tear secretion by eliminating the reflex tearing component using a topical anesthetic,.

### 2. Equipment Required

- Schirmer Tear Test Strips: Sterile, standardized filter paper strips (5 x 35 mm) packaged in pairs,.

- Millimeter Ruler: To measure the wet portion (often printed on the box or included in the strip packaging),.

- Tissue: To blot excess tears or dry the eyelids,.

- Topical Anesthetic (Optional): Required only if performing the Basic Secretion Test,.

- Timer/Watch: To time the 5-minute duration,.

### 3. Detailed Procedure

#### Preparation

- Patient Comfort: Seat the patient comfortably with their head resting against the headrest. Dimming the room lights can help reduce patient discomfort and reflex tearing due to light sensitivity,.

- Strip Preparation: Before opening the sterile package, bend the rounded (wick) end of the strips at the pre-marked notch to approximately a 120° angle. Open the package carefully, handling the strips only by the non-wick (square) end to maintain sterility,.

- Lid Preparation: Gently dry the outside of the closed eyelids with a tissue to prevent skin oils or sweat from interfering with the test.

#### Insertion

- Positioning: Instruct the patient to look up. Gently pull the lower eyelid down and temporally,.

- Placement: Hook the bent end of the strip over the lower lid margin. Place it at the junction of the temporal (outer) one-third and central one-third of the lower eyelid,.

  - Critical Step: Ensure the strip touches the palpebral conjunctiva (inside the lid) but does not touch the cornea, as this will cause pain and excessive reflex tearing,.

- Duration: Release the eyelid. Instruct the patient to gently close their eyes for 5 minutes.

  - Note: Closing the eyes is generally preferred to prevent blinking, which acts as a pump, though some protocols allow eyes to remain open and blinking normally,,.

  - Patient Instruction: Tell the patient to relax, breathe normally, and not to squeeze their eyes shut,.

#### Measurement

- Removal: After 5 minutes, gently remove the strips.

- Reading: Measure the length of the wetted area from the notch (bend) downwards in millimeters,.

### 4. Interpretation of Results

- Normal: Greater than 10 mm of wetting in 5 minutes is generally considered normal,,.

- Borderline: Between 5 mm and 10 mm.

- Dry Eye (Tear Deficiency): Less than 5 mm of wetting indicates a significant aqueous tear deficiency,.

- Note: If the result is 0 mm, it indicates a severe deficiency. If the entire strip is wet before 5 minutes, it indicates excessive reflex tearing,.

### 5. Documentation (Recording Method)

Record the findings for each eye separately (OD for Right, OS for Left).

- Test Type: Specify whether it was Schirmer I (no anesthetic) or Basic Secretion (with anesthetic),.

- Measurement: Record the amount of wetting in millimeters (mm).

- Duration: Confirm the time duration (usually 5 minutes).

Example Recording:

- Schirmer I: OD 12mm / OS 8mm (5 mins).

- Schirmer (with anesthetic): OD 6mm / OS 5mm.

### 6. Patient Communication (What to tell the patient)

- "This test measures your tear production. It might feel a little scratchy, like an eyelash is in your eye, but it will not be painful.".

- "Please keep your eyes gently closed and try to relax for 5 minutes. I will remove the paper strips when the time is up.".